# Water Shortage Emergency Response Plan

# For Small Public Drinking Water Systems

For more information or additional copies of this guidance document, call (402) 471-0521, via e-mail at DHHS.drinkingwater@nebraska.gov or write to:

DHHS Drinking Water Program

1200 “N” Street, Suite 400

PO Box 98922

Lincoln, Nebraska 68509-8922

Contents

PART 1: GUIDANCE AND INSTRUCTIONS………………………………………1 - 12

Introduction: Protecting Public Health

The Requirement for a Water Shortage Response Plan

How to Use this Guidance Document

Section 1.  Events that Cause Water Shortages

Section 2.  Evaluate Supply and Demand

Section 3.  Defining Stages and Criteria of a Water Shortage

Section 4.  Alternate or Emergency Water Sources

Section 5.  Effective Communication

Section 6.  Demand Reduction

Section 7.  Water Shortage Response Actions

PART 2: PLANNING TEMPLATE………………………………………………….13 - 23

Introduction

How to Use this Template

Section 1.  Events that Cause Water Shortages

Section 2.  Evaluate Supply and Demand

Section 3.  Defining Stages and Criteria of a Water Shortage

Section 4.  Alternate Water Sources

Section 5.  Effective Communication

Section 6.  Demand Reduction Alternatives

Section 7.  Water Shortage Response Actions

Part 1:  Guidance and Instructions

Introduction: Protecting Public Health

Safe and reliable drinking water is vital to every community. Being prepared and knowing how to respond to a water shortage is an essential part of managing a drinking water system.

Most public water systems have experienced routine types of water shortages such as those caused by pipe breaks, pump malfunctions, and power outages.  These types of shortages are typically manageable.  More serious water shortages result from drought, or storm events that directly affect the water source.  These can drastically affect the water system and the community that depends on it.

Water shortage response planning is a process water systems can use to identify events that could cause water shortages and specific actions to reduce customer demand.  A good water shortage response plan can help systems minimize water shortages, or avoid them altogether.  However, a water shortage plan is only for use during unusual events.  It is not a substitute for good water supply development and sound conservation programs.

The Requirement for a Water Shortage Response Plan

If a public water system experiences a water shortage, or anticipates experiencing one in the next several years, the Nebraska Department of Health and Human Services, Division of Public Health (Department) recommends that a public water system prepare a water shortage response plan as part of a water system emergency response plan.  This guidance document can help water systems prepare for a water shortage.

In addition, drought-related financial assistance programs sometimes require water systems to have a water shortage response plan to qualify for funding.

Any water system can use this guidance document.  However, it is primarily for small water systems to complete a water shortage response plan to use when preparing to address potential water shortage events or seeking financial assistance for drought-related projects.

How to Use this Guidance Document

Developing a water shortage response plan takes time and effort.  This guidance document can make the job easier and help create a detailed plan for responding to a water shortage.  Each water system can modify it to meet specific needs.  Smaller water systems should consider each section and use what is relevant for the type, size and complexity of their facility.

This guidance document can be used independently, but it is designed for use with the Department’s Emergency Response Planning Template for Public Drinking Water Systems.

This guidance document has two parts:

Part 1 discusses important water shortage response planning elements and provides instructions and examples to help complete Part 2.  You can also use Part 1 as an educational tool to help water system staff understand the key components needed for a well thought-out plan.

Part 2 is a template for creating your own plan.  You may use the template in its original form or modify it to meet your system’s needs.  An electronic copy of this guidance document, including templates allowing you to fill in the blanks with a computer, is on the Department’s web site.  (See Water Shortage Response Plans for Small Public Drinking Water Systems.)

Do not submit your plan to the Department unless it is requested.  The Department may request the plan as part of a water system plan or a funding requirement.

Be sure to store your plan in a safe and secure location.  The Department recommends you store one copy on-site and another off-site to ensure it is available if you are unable to access your offices or facilities.

Section 1.
Events that Cause Water Shortages

Water shortages occur for a variety of reasons, including:

- Natural disasters

- Extreme customer usage

- Equipment failures

- Leakage

- Shallow wells or inadequate source water

Each type of event causes different situations, including complete service disruption, low pressure, air or sediments in the supply or backflow contamination.  You should evaluate a variety of events and the ways they could affect your water system and its infrastructure.  Remember, a water shortage may affect the entire water system or only isolated sections.  Include these evaluations in your water shortage response plan along with procedures for responding to specific events.

What can cause a water shortage?

Consider common natural disasters and other factors when developing a water shortage response plan, including:

Drought:  Droughts are a major issue in and can have devastating effects on water supplies.  During normal years, peak summer demands can double and even triple water use.  These same demands during low water years can lead to water shortages.  A combination of environmental factors affects drought severity.  These factors change over time and include rainfall, temperature, snow pack and length of drought.  Compared to other natural disasters, drought has a relatively slow onset and is easier to anticipate.  However, drought-related problems can be the most difficult to resolve because they typically involve a shortage in the actual water supply.

High Winds:  Wind storms often disrupt power and damage water system facilities, creating a disruption in the supply.

Equipment Failure:  Drinking water systems must continuously evaluate facilities and replace them before a water shortages or failure occurs.  For example, aging well pumps may cause an emergency water shortage if they need replacement, and go without attention so long that they fail, or run low on water due to drought.

Construction Accidents:  A utility must be aware of construction in and around the water system and be prepared to respond quickly if an accident occurs.  For example, if a contractor damages a water line, the water system may need to be shut down for repair.  If the response is not timely and effective, this kind of incident could turn into a serious water outage.  The water system may lose pressure, resulting in serious backflow incidents that contaminate the water.  The response may include asking customers to curtail water use until repairs are made.

Define the Types of Events that May Affect the Water System

A water system may be vulnerable to various types of events that could cause a water shortage.  Understanding and evaluating these vulnerabilities is an important part of preparing a water shortage response plan.  Start by defining the events most likely to affect your water system.  Next, consider the probability of them happening and then assign a risk of high, medium or low.  Consider that some events cause an immediate water shortage, without notice, such as an earthquake or equipment failure.  Other events can be anticipated, and planning may reduce or avoid the impact altogether, as in drought.

Immediate water shortages require immediate actions to fix the problem or bring in an alternative source.  However, anticipating a drought may require water conservation activities to avoid a future water shortage.

Sample of Events that Cause Water Shortages at XYZ Water System

| Event | Probability or Risk (High – Med – Low) | Immediate or Anticipated Event | Comments |
| --- | --- | --- | --- |
| Drought | High | Anticipate | Need to plan for decrease in well or source yield during dry summers. Implement water conservation measures to minimize well impacts. |
| High Winds | High | Anticipate | Water system is vulnerable to high wind events. Power is disrupted. Make adjustment prior to wind events. |
| Ice Storm | Med | Immediate | Minor damage caused in December 1996. Broken pipes and damaged pump house. |
| Flood | Low | N/A | Water system located in an area not vulnerable to flooding. |

Section 2.

Evaluate Supply and Demand

To prepare a water shortage response plan, a water system must evaluate and compare supply and demand.  To do this, a water system must have both supply and demand information.

Ideally, a water system should be able to determine whether a water shortage is likely and be able to predict the magnitude of the shortage.  This determination is complicated because of the difficulty in estimating available supplies.

Source and service meters present the best way to obtain the necessary data to analyze supply and demand.  However, many small water systems do not have service meters and some do not have source meters.  Therefore, water systems must use the best available information to make an estimate.

The primary objective is determining whether the water system is vulnerable and faces the possibility of a shortage.  Next, the water system must estimate the magnitude of a potential shortage using the best information available.  Answer the basic question:  “How likely is it the water system will use more water than it can produce?”  If it seems likely, consider conservation measures to reduce usage within the limits of the supply.  If the anticipated shortage is more severe, such as a well failure, consider alternative water supplies.

Evaluate the Source of Supply

Knowing the source of supply is critical to understanding if environmental conditions or demand could lead to a water shortage.  What it is capable of producing?  How it is constructed?  What is its geological and environmental make-up?

To truly assess the supply, know the basics:  How deep is the well?  How big is the well pump?  What does it produce?  What kind of geologic characteristics does the well have?  Most of this information is in the Well Drillers Log completed when the well was drilled.  If you need a copy of the report, call the Nebraska Department of Natural Resources at (402) 471-2363.

Example: Source of Supply Information

| Source ID | Well Depth | Pump Size | Water Level | Well Test | Water Right Info | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| SO3202 | 98 feet | Goulds 3 hp | Static Level 25 feet | 20 gpm 6 hr, 10’ drawdown | Qi 25 gpm Qa 10 acre feet | Geologic make-up results in significant draw down during summer. |

| Questions to Help You | Answer |
| --- | --- |
| How much water can the well pump produce in 1 hour? |  |
| How much water can the well pump produce in a 24-hour period? |  |

Evaluate Consumption (Demand)

If you have source and service meters and you keep good records, understanding the supply and demand is relatively simple.  However, if you do not have source or service meters, there are other ways to measure demand:

A.	If there is an elapsed time meter on the well pump and the pumping capacity is known for a given pressure, it is possible to determine the amount of water consumed over a given period of time.

B.	If there is a storage reservoir, it is possible to determine consumption by shutting down the pumps and measuring the drawdown over a period of time.  Ideally, the reservoir is recorded, the pumps are shut down for 24 hours, and the drawdown is measured for the 24-hour period.  Repeat once a week, for two or three weeks, and average the results.  If storage is inadequate for the 24-hour period, you may have to reduce the time to 12 hours and test more frequently.

Compare Supply and Consumption

Compare the supply and consumption information by answering the following questions:

| Questions to Help You | Yes or No |
| --- | --- |
| Can the source and pumping capability meet average day demand? |  |
| Can the source and pumping capability meet peak day demands? |  |
| Is the water in the well lower than normal? |  |
| Is there more than normal or significant drawdown in the storage reservoir during times of average and/or peak day demand? |  |
| Does consumption ever exceed the supply resulting in water shortage problems? |  |
| Think about if the supply were reduced by 10%, 20% and 30%. Where is the point at which demand could not be met? | What percent? |
| Is it likely the water system will experience a water shortage? If yes, do you consider the magnitude of the shortage severe, moderate or minor? |  |

Section 3.

Defining Stages and Criteria of a Water Shortage

The severity of water shortages varies widely depending on the type of event.  Defining stages of severity can significantly aid in determining appropriate response actions.  For example, anticipating events that may affect your system, and estimating how severe the effects would be, makes it easier to respond and communicate with others.  It also helps water system personnel keep their response balanced and effective.

In classifying the severity of a water shortage, define as many stages and criteria for each stage as you find useful.  Begin by defining the stages and then list criteria that describe them.  The following is a four-stage example for a water system supplied by groundwater.  This is just an example; you may choose to classify water shortages another way.  Very small water systems may prefer using two or three stages.

Stage I – Advisory Actions:  Water system manager or operator knows there is potential for a water shortage based on projected drought conditions, a drought declaration or other emergency prediction.  In this situation, it is not likely a water shortage will happen in the near future but it is important to begin responding to avoid or minimize future impacts.

###### Example Description and Criteria: Stage 1 – Advisory

Description: The XYZ water system considers the following a Stage I water shortage: - Projected drought conditions or a drought declaration, but prior to noticing impacts. - Major storm predictions. - Well water level noticeably lower than in recent years for the given time of the year. Note: The water system has specific response activities identified for these types of shortages, including public awareness messages about water conservation. If the system determines problems are likely, the situation will progress to Level II.

Stage II – Voluntary Actions:  The water system anticipates conditions likely result in a minor water shortage.  Minor is less than 5 percent of normal supply.  It is important the water system communicate with customers about the event and the voluntary actions they can take to help minimize the effects.

Example Descriptions and Criteria:  Stage II – Voluntary

Description: The XYZ water system considers the following a Stage II water shortage: - Drought declaration with worsening conditions projected. - Drought, with a slight but noticeable and continuing decline of water level in the well.

Stage III – Mandatory Actions:  The water system experiences significant supply reduction or mechanical problems where disruption in supply is inevitable.  Water system personnel are directed to implement mandatory water conservation or curtailment measures to reduce demand.

###### Example Descriptions and Criteria:  Stage III – Mandatory

Description: The XYZ water system considers the following a Stage III or actual emergency: - Verified 10 percent or greater reduction in source of supply including well level decrease below normally observed conditions. - Evidence of pressure problems. - Short-term power outage where water system is running on a generator or using storage. - Pump or other equipment failure, but water system is able to produce and supply a partial supply of water. - It is evident severe drought conditions will affect well yield.

Stage IV – Emergency Actions:  The water system experiences loss of water because of major damage or reduction of source supply from a natural disaster, accident, act of terrorism or severe drought conditions, i.e., the well is out of water.  These incidents require the water system to implement its emergency response plan immediately.  It may be necessary to immediately notify affected parties.  Immediately issuing health advisories and declaring a water supply emergency are critical to protect public health.  These events often take several days or weeks to resolve before the water system returns to normal operation.

Example Description and Criteria:  Stage IV – Emergency

Description: The XYZ water system considers the following events to be Stage IV emergencies, requiring implementation of its emergency response plan: • Damages shut down the water system or affect its sources, lines and so on. • Well runs dry as a result of drought or supply is unable to keep up with pump. • Flood infiltrates water system facilities and sources. • Storm significantly damages power grid and water system facilities.

Section 4.

Alternate or Emergency Water Sources

Water shortages may require the water system to get water from an alternate source or switch to an emergency back-up source to meet basic community needs.  All public water systems should plan ahead to provide alternate safe water or switch to an approved back-up source, if one is available.  It is important to evaluate potential alternate and emergency water supplies ahead of time to ensure the water is safe and the supply is available.

Sources a water system may use when primary and seasonal sources cannot meet demands are “emergency sources.”  They may include back-up or emergency wells, surface water. Emergency sources are used only as required by extreme, mostly unpredictable circumstances.  A water system that anticipates using an emergency source should plan and act well in advance of any need.  As part of water shortage response planning, the water system should test these sources.

The water system also may consider establishing an inter connection with an approved water supply that could benefit both water systems in an emergency.  Discuss this possibility with adjacent water systems.  Other emergency sources to consider are bottled water suppliers or a local tanker truck that could bring water in.

Example:  Inter-connection to an Adjacent Water Supply System

| Water Systems within ¼-mile of our Water System | Feasibility of Connecting |
| --- | --- |
| There is one water system located within 1/4-mile of the XYZ water system. The XYZ distribution water system is within 1,000 feet of the other water system. | The water system has discussed installing an inter-connection with the adjacent water supply. The water system is willing but, at this time, cannot assist financially unless there is an actual emergency. The cost of the project is about $10,000 to install pipe and an intertie connection. In the event of a potential water shortage, the XYZ water system will look for funding to construct the intertie. |

Example:  Switching to a Back-up Well

| Source Description | Well ID | Required testing before bringing on-line | Special considerations and procedures |
| --- | --- | --- | --- |
| The XYZ water system has an emergency back-up well that may be used in the event of a water shortage. The well will not supply the entire water system but will augment current demand. | SO# | Must sample for coliform and nitrates Sample for IOC is recommended | Notify the Department Flush and disinfect well |

Example:  Alternate Source(s) of Water

| Alternative Sources | Names | Phone | Availability | Is the water safe for drinking? |
| --- | --- | --- | --- | --- |
| Bottled water suppliers | Bottled Water Inc. | (360) 222-2222 | Up to 1,000 gallons in one-gallon jugs within 24 hours | Yes |
| Tanker trucks in the area available to deliver bulk water | Fred Jones local dairy truck | (509) 333-3333 | 5,000 gallons in less than 6 hours | No |

Section 5.

Effective Communication

Effective communications with customers is integral to successful water system operations and should be part of a water system’s overall emergency planning.  During a water shortage, customers may be concerned or upset.  It’s important to earn their trust early.  Communicate with customers as early and directly as possible about potential shortages, especially anticipated ones, such as a drought-related shortage.  This may pave the way to better cooperation when it’s time to implement water conservation or curtailment measures, especially if they become mandatory.

During a drought, it is especially important to understand and plan messages carefully.  Coordinate with local and state health officials or a third-party technical assistance provider to develop key messages and ways to deliver them.  Strive for clarity, and avoid jargon and technical terms.

Once a shortage is over, promptly deliver the good news to customers and thank those who assisted in resolving the event.  By asking customers to evaluate how you did and to recommend what would work better for them, you can prepare better for the next situation.

As part of the water shortage response plan, develop key messages and make sure field and office staff knows how to deal with the media and questions from customers and the public.  It may be necessary to establish protocols for field and office staff to defer questions respectfully to a spokesperson.

Example:  Key Water Shortage Messages

Develop possible messages in advance, and update them as an event develops: - There is potential for a water shortage this year due to drought conditions. We are taking this seriously and will be communicating with you regularly. - Our primary concern is protecting our customers’ health and the quality of the water being supplied by keeping the water system operational. - What we know right now is ____________________. - “We respectfully request customers to conserve water” by implementing water conservation measures. - The potential exists for a less-than-normal water supply. Customers are advised that water-use restrictions may become necessary to minimize or avoid water supply problems. - “The current drought situation is beginning to affect our water supplies. Well levels are declining at a higher rate than normal. Please use water wisely during this time.” - Please be advised that everyone is required to conserve water in the following ways:

Section 6.

Demand Reduction

Demand reduction measures, including water conservation and water use curtailment, are a key component of a water shortage response plan.  It is important the water system develop a list of possible actions it can take in response to a potential or real water shortage.

All water systems should have a water conservation program encouraging wise water use.  Conservation measures can be implemented well in advance of a potential shortage and may be the key to avoiding a shortage altogether.  Develop a water conservation program with clear measures that can be implemented over a long period of time.

An actual water shortage may require reducing water usage, so identify curtailment measures in advance.  Possible measures include restrictions on landscape watering, car washing, filling swimming pools and hot tubs, and other nonessential activities such as cleaning driveways and sidewalks.  The water system should develop and formally adopt measures through ordinance, resolution or by-laws.

As part of this effort, consider ways to inform customers about conserving water or curtailing water use.  Examples include door-to-door postings, phone contact, posting signs in visible community areas and contacting the news media.  Use the key messages you developed in advance to ensure communications are clear and you say want you really want to say.

Example:  Water Conservation Measures

| Water Conservation Measures | Actions necessary for implementation |
| --- | --- |
| Limit watering to even-number days for west end residences and odd-number days for east end residences. Implement conservation-based rates on June 15. | - Deliver conservation brochures and door hanger regarding irrigation times. - Send a rate notice with the May bill. |

Example:  Curtailing Water Use Measures

| Water Curtailment Measures | Actions necessary for implementation |
| --- | --- |
| Restrict outside water usage such as watering lawns, washing cars and so on. Request curtailment of inside usage. | - Develop door hanger with curtailment messages and post on customer doors. - Contact the local radio news station to announce curtailment message. - Monitor water system usage and spot check meter usage, if time is available. |

Section 7.

Water Shortage Response Actions

Numerous events could cause a water shortage, depending on a water system’s size, complexity, type of source and geographic location.  As discussed before, likely causes of a water shortage are power outages, transmission or distribution line breaks, earthquakes, source pump failures, loss of water in wells, droughts, and floods.  In any of these situations, the priority is protecting the people who use the water.  Therefore, the core of the response plan is specific actions the system can take to respond to different events.

The following pages provide a format and examples for your water shortage response plan.  However, this is only the starting point.  Every water system is unique.  It is important to prepare for any situation that could affect your system, even those not covered here.

Example: Water Shortage Response Action Plan Worksheet

| Stage | Criteria | Actions | Messages |
| --- | --- | --- | --- |
| Stage 1 Advisory | Potential for drought Drought declaration Well level slightly lower (less than 10 percent) than in recent years on the current date. | Advise water system customers of situation. Advise staff of situation and request they implement water conservation objectives during job activities. Begin monitoring well level every two weeks. Keep up to date weekly on drought monitor at: http://www.drought.unl.edu/dm/monitor.html | “We may soon experience less-than-normal water supplies. Customers are advised that water use restrictions may become necessary to make sure we don’t run out of water.” “Please use water wisely. Refer to water conservation education materials”. |
| Stage 2 Voluntary | Drought declaration – area is in a moderate to severe drought condition and conditions are expected to worsen. Noticeable and continuous decline of water level in the well each week. | - Post advisory and provide Department’s Water Conservation Wheel* to customers. - Request water system customers to conserve water by: - Limiting outside watering to 2 days a week between 7:30 p.m. and 7 a.m. - Washing vehicles on grassy areas. - Following the Department’s Water Conservation Guidelines* 1, 2 and 3: - Indoor water conservation - Outdoor water conservation - Lawn watering guide - Continue monitoring well level once a week. - If well declines more than 10 percent, elevate to stage 3. *Water Conservation Guidelines are online at http://www.dhhs.ne.gov/enh/pws/conindex.htm | “Our water supplies are being affected by the current drought situation.” "Well levels are declining at a rate higher than normal.” “We respectfully request customers to conserve water” by implementing water conservation measures. “Please use water wisely. Refer to water conservation education materials”. |
| Stage 3 Mandatory | System estimates 10 percent or larger reduction in source supply, including decreased well level or below-normal surface water flow. Short term power outage where water system is running on generator or storage. Pump or other equipment failure, but water system is able to produce and supply. A reduced supply of water. Severe drought conditions likely will affect well yield. | Initiate mandatory water use restrictions, such as: - Limiting lawn watering to 1 day per week, less than 2 hours, between 8 p.m. and 6 a.m. - Prohibiting all daytime irrigation. - Limiting outside watering to odd-number days, after 8 p.m. and no more than 1 hour. Implement a water-conservation rate structure. Inform customers about the situation. Monitor well level twice a week at varying intervals. Post notices around neighborhoods. Implement water use rates if well declines below 20 percent of normal. | “Due to a continuing decline in our source water supply, we need to impose mandatory water use restrictions. Until further notice, everyone is required to conserve water in the following ways: (Develop list) |
| Stage 4 Emergency | Well level is unsafe for pumping, pumps are cavitating, or there is air in the distribution system. Natural disaster shuts down the water system or affects sources, lines and so on. Flood inundates water system facilities and sources. Storm significantly damages power grid and water system facilities. | Mandatory rationing. Bring in bottled and trucked water. | “You may obtain water for emergency use from the storage reservoir. We will be available to assist you from ___ to ___, or call…” If you use water from this source, boil it for 3 to 5 minutes and let it cool before using. |

Part 2: Planning Template

Introduction

Preparing an emergency response plan is an essential part of managing a drinking water system.  The Department is making this template available to all public water systems in Nebraska to help them develop such plans.

How to Use this Template

The template follows the outline in Part 1 of this guidance document.  Part 1 discusses key components of emergency planning and provides ways to present information in the plan.  Use Part 1 as a tool to learn about emergency planning.  And then, as you go through the planning process, complete the template on the following pages.

The template is just a guide; you may modify it in any way that works for you – add sections, take them out, or rearrange them if you wish.  You may also use a completely different format for the plan if you find one that works better for the water system.  The sections in the template are:

Section 1. Events that Cause Water Shortages

Section 2. Evaluate Supply and Demand

Section 3. Defining Stages and Criteria of a Water Shortage

Section 4. Alternate Water Sources

Section 5. Effective Communication

Section 6. Demand Reduction Alternatives

Section 7. Water Shortage Response Actions

Section 1.
Events that Cause Water Shortages

List below the events that may cause water system emergencies for your system.  Arrange them from highest to lowest probable risk.  See pages 2-3 of this guidance document for more information.

Events that Cause Emergencies

| Type of Event | Probability or Risk (High – Med – Low) | Immediate or Anticipated Event | Comments |
| --- | --- | --- | --- |

Section 2.
Evaluate Supply and Demand

Use the following tables to define information that will help estimate supply and demand.  See pages 3-5 of this guidance document for more information.

Evaluate the Source of Supply

To truly assess the supply, begin by putting basic information about each water source in the box below.

Source of Supply Information

| Source ID | Well Depth | Pump Size | Water Level | Well Test | Water Right Info | Notes |
| --- | --- | --- | --- | --- | --- | --- |

Answer the questions below to completely understand the make-up and capacity of the supply.  Most of this information is in the Well Drillers Log completed when the well was drilled.  If you need a copy of the report, call the Nebraska Department of Natural Resources at (402) 471-2363.

| Questions to Help You | Answer |
| --- | --- |
| How much water can the well pump produce in 1 hour? |  |
| How much water can the well pump produce in a 24-hour period? |  |
| Does the water right allow for a larger pump to be placed in the well? |  |

Evaluate Consumption (Demand)

If you have a source meter and service meters and you keep good records, understanding the supply and demand is relatively simple.  However, if you do not have source or service meters, there are other ways to measure demand:

A.	If there is an elapsed time meter on the well pump and the pumping capacity is known for a given pressure, it is possible to determine the amount of water consumed over a given time period.

B.	If there is a storage reservoir, it is possible to determine consumption by shutting down the pumps and measuring the drawdown over a period of time.  Ideally, the reservoir is recorded, the pumps are shut down for 24 hours, and the drawdown is measured for the 24-hour period.  Repeat once a week, for two or three weeks, and average the results.  If storage is inadequate for the 24-hour period, you may have to reduce the time to 12 hours and test more frequently.

Compare Supply and Consumption

Compare the supply and consumption information by answering the following questions.

| Questions to Help You | Yes or No |
| --- | --- |
| Can the source and pumping capability meet average day demands? |  |
| Can the source and pumping capability meet peak day demands? |  |
| Is the water in the well lower than normal? |  |
| Is there more than normal or significant drawdown in the storage reservoir during times of average and/or peak day demand? |  |
| Does consumption ever exceed supply resulting in water shortage problems? |  |
| Think about if the supply were reduced by 10%, 20% and 30%. Where is the point at which demand could not be met? | What percent? |
| Is it likely the water system will experience a water shortage? If yes, do you consider the magnitude of the shortage severe, moderate or minor? |  |

Section 3.
Defining Stages and Criteria of a Water Shortage

Defining the stages of a water shortage should be discussed among water system personnel, but the person in charge ultimately must decide what they will be.  The information for making this decision accumulates over time, and may cause changes in the assessment of severity.  To assist in the decision-making process, identify and define the stages and the specific events that trigger them.  See pages 5-6 for more information.

Stage I – ____________________  (Definition)

Description: Criteria:

Stage II – ____________________  (Definition)

Description: Criteria:

Stage III – ____________________  (Definition)

Description: Criteria:

Stage IV – ____________________  (Definition)

Description: Criteria:

Section 4.
Alternate Water Sources

Identifying and planning for alternate water sources is an important part of responding to a water shortage.  The water system may already have completed this exercise as part of its overall emergency response plan.  If so, use the same information below, or refer to the emergency response plan.  See pages 7-8 for more information.

Inter connection to Adjacent Water Supply System

| Water Systems within ___ mile(s) of our Water System | Feasibility of Connecting |
| --- | --- |

Switching to Back-up Well

| Source Description | Well ID | Required testing before bringing on-line | Special considerations and procedures |
| --- | --- | --- | --- |

Alternate Source(s) of Water

| Alternate Sources | Names | Phone | Availability | Is the water safe for drinking? |
| --- | --- | --- | --- | --- |

Section 5.
Effective Communication

Communication with customers, the news media, and the general public is a critical part of water shortage response planning and actions.  Develop key messages that are clear and understandable to the customer.  See page 8 for more information.

Key Messages

Develop possible messages in advance, and update them as the shortage develops:

Section 6.
Demand Reduction Alternatives

It is important to develop a list of possible actions the water system can take to respond to a potential or actual water shortage.

Water Conservation Measures

| Water Conservation Measures | Actions necessary for implementation |
| --- | --- |

Curtailing Water Use

| Water Curtailment Measures | Actions necessary to implementation |
| --- | --- |

Section 7.
Water Shortage Response Actions

Define specific water shortage response actions on the worksheet provided, or use your own.  This is the core of your plan. Use the examples in Part 1 as a starting point.

| Stage | Criteria | Actions | Messages |
| --- | --- | --- | --- |

| Stage | Criteria | Actions | Messages |
| --- | --- | --- | --- |

| Stage | Criteria | Actions | Messages |
| --- | --- | --- | --- |